COMPUTER NETWORKS LABORATORY

Lab Report

Supernetting

Route Aggregation and CIDR

Lab File: Supernetting_1.pkt

Course: Computer Networks

Date: March 2026

# 1. Objective

To understand and apply supernetting (CIDR) by aggregating multiple contiguous network addresses into a single summarized route, reducing routing table size in Cisco Packet Tracer.

# 2. Background Theory

Supernetting (route aggregation) is the process of combining multiple contiguous IP networks into a single larger block. CIDR introduced prefix notation to overcome classful address exhaustion.

## 2.1 How to Supernet

Given four /24 networks (192.168.0.0 through 192.168.3.0), convert to binary and find the common prefix:

192.168.00000000.0  ->  192.168.0.0/24

192.168.00000001.0  ->  192.168.1.0/24

192.168.00000010.0  ->  192.168.2.0/24

192.168.00000011.0  ->  192.168.3.0/24

Common prefix = 22 bits -> Supernet: 192.168.0.0/22

## 2.2 Benefits of Summarization

- Reduces routing table entries — improves scalability

- Decreases router processing overhead

- Faster convergence — fewer LSAs/updates propagated

- Limits routing update scope to relevant areas

# 3. Network Topology & IP Scheme

| Network | Prefix | Purpose |
| --- | --- | --- |
| 192.168.0.0 | / 24 | LAN Segment 1 |
| 192.168.1.0 | / 24 | LAN Segment 2 |
| 192.168.2.0 | / 24 | LAN Segment 3 |
| 192.168.3.0 | / 24 | LAN Segment 4 |
| 192.168.0.0 | / 22 | Aggregated Supernet |

# 4. Configuration

## 4.1 EIGRP Route Summarization

Router(config-router)# summary-address eigrp 100 192.168.0.0 255.255.252.0

## 4.2 OSPF Area Range

Router(config-router)# area 0 range 192.168.0.0 255.255.252.0

# 5. Observations

- Before: upstream router had 4 separate /24 routes.

- After: upstream router shows only 192.168.0.0/22.

- All four subnets remained reachable via the aggregated prefix.

- Routing table reduced by 75% (4 entries to 1) for those networks.

- Summarizing router installed Null0 route to prevent routing loops.

## 5.1 Before vs After

BEFORE: O 192.168.0.0/24, O 192.168.1.0/24, O 192.168.2.0/24, O 192.168.3.0/24

AFTER:  O 192.168.0.0/22 [110/65] via ...

O 192.168.0.0/22 is a summary, Null0  (loop prevention)

# 6. Conclusion

Supernetting via CIDR is essential for scalable routing. Aggregating four /24 networks into a single /22 demonstrated a 75% reduction in routing entries. Proper IP address design — contiguous, power-of-2 aligned blocks — is essential for effective summarization and is a critical skill for network engineers.

# 7. References

- RFC 1519 — CIDR (Classless Inter-Domain Routing)

- Cisco Route Summarization Configuration Guide

- Forouzan, B. A. (2012). Data Communications and Networking. McGraw-Hill.