COMPUTER NETWORKS LABORATORY

Lab Report

Default Routing

Gateway of Last Resort Configuration

Lab File: default.pkt

Course: Computer Networks

Date: March 2026

# 1. Objective

To configure and verify default routes (gateway of last resort) on Cisco routers and understand when and why default routing is used in real-world scenarios.

# 2. Background Theory

A default route is a routing table entry used when no specific route matches the destination IP address. It is the catch-all route that forwards packets to a next-hop when no more specific path is known.

## 2.1 Default Route Properties

- Network: 0.0.0.0, Subnet Mask: 0.0.0.0 (notation: 0.0.0.0/0)

- Lowest specificity — matches any destination

- Ideal for stub networks, branch offices, internet gateways

## 2.2 Configuration Command

Router(config)# ip route 0.0.0.0 0.0.0.0 [next-hop-ip]

## 2.3 Propagating via OSPF

Router(config-router)# default-information originate

# 3. Network Topology & IP Scheme

| Interface | IP Address | Subnet Mask |
| --- | --- | --- |
| Edge Fa0/0 (LAN) | 192.168.1.1 | 255.255.255.0 |
| Edge Se0/0 | 203.0.113.1 | 255.255.255.252 |
| Core Se0/0 | 203.0.113.2 | 255.255.255.252 |
| Core Fa0/0 | 10.0.0.1 | 255.255.255.0 |

# 4. Procedure

- Open default.pkt in Cisco Packet Tracer.

- Configure default route on Edge router: ip route 0.0.0.0 0.0.0.0 203.0.113.2

- Verify routing table: show ip route — look for S* 0.0.0.0/0

- Test connectivity from PC0 to remote networks.

- Verify Gateway of Last Resort is set correctly.

# 5. Observations

- Edge routing table showed only 2 entries: connected LAN + default route.

- S* 0.0.0.0/0 via 203.0.113.2 appeared as Gateway of Last Resort.

- All unknown destination traffic was forwarded to the Core router.

- Bidirectional connectivity required return routes on Core router.

## 5.1 Edge Router Routing Table

C    192.168.1.0/24 is directly connected, FastEthernet0/0

S*   0.0.0.0/0 [1/0] via 203.0.113.2

Gateway of last resort is 203.0.113.2 to network 0.0.0.0

# 6. Results and Analysis

| Router | Route Entries | Method |
| --- | --- | --- |
| Edge Router | 2 | Default + Connected |
| Core Router | 4+ | Static / Dynamic |

# 7. Conclusion

Default routing is a powerful technique for stub network scenarios. A single 0.0.0.0/0 route allows edge routers to forward all unknown traffic toward the network core without knowing every destination. This reduces routing table size, simplifies configuration, and is ideal for single-exit-point designs such as branch offices or internet edge routers.

# 8. References

- Cisco IOS IP Routing Configuration Guide

- RFC 1122 — Requirements for Internet Hosts

- Lammle, T. (2020). CCNA Certification Study Guide. Sybex.