COMPUTER NETWORKS LABORATORY

Lab Report

EIGRP Routing Protocol

Enhanced Interior Gateway Routing Protocol

Lab File: EIGRP.pkt

Course: Computer Networks

Date: March 2026

# 1. Objective

To configure and verify EIGRP on Cisco routers, observe neighbor discovery, DUAL algorithm operation, and verify load balancing capabilities using Cisco Packet Tracer.

# 2. Background Theory

EIGRP is an advanced distance-vector protocol developed by Cisco (later open standard RFC 7868). It uses the DUAL algorithm for guaranteed loop-free, fast-converging routing.

## 2.1 Key Concepts

- DUAL Algorithm: Guarantees loop-free paths and fast convergence.

- Successor: Primary route (lowest feasible distance).

- Feasible Successor: Pre-computed backup route.

- Composite Metric: Based on Bandwidth, Delay, Reliability, Load, MTU.

- Autonomous System (AS): Routers must share the same AS number.

## 2.2 EIGRP Packet Types

| Type | Name | Purpose |
| --- | --- | --- |
| Hello | 5 | Neighbor discovery & keepalive |
| Update | 1 | Routing information exchange |
| Query | 3 | Request for route information |
| Reply | 4 | Response to Query |
| ACK | 2 | Reliable delivery acknowledgment |

# 3. Network Topology & IP Scheme

| Interface | IP Address | Subnet Mask |
| --- | --- | --- |
| R0 Fa0/0 | 192.168.1.1 | 255.255.255.0 |
| R0 Se0/0 | 172.16.0.1 | 255.255.255.252 |
| R1 Se0/0 | 172.16.0.2 | 255.255.255.252 |
| R1 Fa0/0 | 192.168.2.1 | 255.255.255.0 |
| R2 Fa0/0 | 192.168.3.1 | 255.255.255.0 |

# 4. Configuration

Router(config)# router eigrp 100

Router(config-router)# network 192.168.1.0

Router(config-router)# network 172.16.0.0

Router(config-router)# no auto-summary

## 4.1 Verification

show ip eigrp neighbors

show ip eigrp topology

show ip route eigrp

# 5. Observations

- EIGRP neighbor adjacency formed within seconds.

- Topology table showed Successor and Feasible Successor routes.

- show ip route displayed D routes with AD of 90.

- When primary link failed, Feasible Successor was instantly promoted.

- EIGRP used multicast 224.0.0.10 for Hello and Update packets.

## 5.1 Topology Table Sample

P 192.168.2.0/24, 1 successors, FD is 2172416

via 172.16.0.2 (2172416/28160), Serial0/0  <- Successor

# 6. EIGRP vs OSPF Comparison

| Feature | EIGRP | OSPF |
| --- | --- | --- |
| Algorithm | DUAL | Dijkstra SPF |
| Admin Distance | 90 (internal) | 110 |
| Convergence | Very Fast | Fast |
| Load Balancing | Equal + Unequal | Equal only |
| Standard | Cisco (open) | Open (RFC 2328) |

# 7. Conclusion

EIGRP proved to be an efficient, fast-converging routing protocol. Its DUAL algorithm provides instant failover using pre-computed Feasible Successors. Unequal-cost load balancing maximizes link utilization, making EIGRP an excellent choice for Cisco-based networks.

# 8. References

- RFC 7868 — EIGRP

- Cisco EIGRP Configuration Guide

- Odom, W. (2020). CCNA 200-301 Official Cert Guide. Cisco Press.