COMPUTER NETWORKS LABORATORY

Lab Report

OSPF Routing Protocol

Open Shortest Path First — Dynamic Routing

Lab File: OSPF.pkt

Course: Computer Networks

Date: March 2026

# 1. Objective

To configure and verify OSPF (Open Shortest Path First) dynamic routing on Cisco routers, observe neighbor adjacency formation, and verify optimal path selection using Dijkstra's algorithm.

# 2. Background Theory

OSPF is a link-state interior gateway protocol (IGP) defined in RFC 2328. It is widely used in enterprise networks due to its scalability, fast convergence, and support for VLSM.

## 2.1 OSPF Packet Types

| Type | Name | Purpose |
| --- | --- | --- |
| 1 | Hello | Neighbor discovery & keepalive |
| 2 | DBD | Database Description summary |
| 3 | LSR | Link State Request |
| 4 | LSU | Link State Update |
| 5 | LSAck | Link State Acknowledgment |

## 2.2 OSPF Cost Metric

Cost = Reference Bandwidth / Interface Bandwidth. Default reference bandwidth is 100 Mbps. Fast Ethernet cost = 1; Serial (T1) cost ≈ 64.

# 3. Network Topology & IP Scheme

| Interface | IP Address | Subnet Mask |
| --- | --- | --- |
| R0 Fa0/0 (LAN) | 192.168.1.1 | 255.255.255.0 |
| R0 Se0/0 | 10.0.0.1 | 255.255.255.252 |
| R1 Se0/0 | 10.0.0.2 | 255.255.255.252 |
| R1 Fa0/0 | 192.168.2.1 | 255.255.255.0 |
| R2 Fa0/0 | 192.168.3.1 | 255.255.255.0 |

# 4. Configuration

## 4.1 OSPF Configuration on Each Router

Router(config)# router ospf 1

Router(config-router)# network 192.168.1.0 0.0.0.255 area 0

Router(config-router)# network 10.0.0.0 0.0.0.3 area 0

## 4.2 Verification Commands

show ip ospf neighbor

show ip route

show ip ospf database

# 5. Observations

- OSPF Hello packets sent every 10 sec (Ethernet) / 30 sec (serial).

- Neighbor adjacency reached FULL state after LSA exchange.

- Routing table shows O routes for remote networks.

- Ping from PC0 to PC2 was successful with RTT < 5ms.

## 5.1 Sample Routing Table

O  192.168.2.0/24 [110/65] via 10.0.0.2, Serial0/0

O  192.168.3.0/24 [110/129] via 10.0.0.2, Serial0/0

# 6. Conclusion

OSPF successfully established neighbor relationships and distributed routing. The SPF algorithm computed optimal paths based on cumulative cost. OSPF's link-state nature and hierarchical area design make it highly scalable for enterprise networks.

# 7. References

- RFC 2328 — OSPF Version 2

- Cisco IOS OSPF Configuration Guide

- Lammle, T. (2020). CCNA Certification Study Guide. Sybex.