COMPUTER NETWORKS LABORATORY

Lab Report

Lab 8 — NAT and PAT

Network Address Translation Configuration

Lab File: lab8.pkt

Course: Computer Networks

Date: March 2026

# 1. Objective

To configure and verify Static NAT, Dynamic NAT, and PAT (Port Address Translation) on Cisco routers, enabling private network hosts to access the internet using public IP addresses.

# 2. Background Theory

NAT modifies IP address information in packet headers as they pass through a router, allowing private (RFC 1918) addresses to be translated to routable public addresses.

## 2.1 Types of NAT

| Type | Description | Use Case |
| --- | --- | --- |
| Static NAT | One-to-one permanent mapping | Servers needing fixed public IP |
| Dynamic NAT | One-to-one from a pool (temporary) | Multiple clients with IP pool |
| PAT / Overload | Many-to-one using port numbers | Most home/enterprise internet |

## 2.2 Private Address Ranges (RFC 1918)

- 10.0.0.0/8

- 172.16.0.0/12

- 192.168.0.0/16

## 2.3 NAT Terminology

- Inside Local: Private IP of internal host.

- Inside Global: Public IP representing the host externally.

- Outside Global: Actual external destination IP.

# 3. Network Topology & IP Scheme

| Device | IP Address | Type |
| --- | --- | --- |
| PC0 | 192.168.1.10 | Inside Local |
| PC1 | 192.168.1.11 | Inside Local |
| PC2 | 192.168.1.12 | Inside Local |
| Router Inside | 192.168.1.1 | Inside Interface |
| Router Outside | 203.0.113.1 | Outside Interface |
| Web Server | 8.8.8.8 | Outside Global |

# 4. Configuration

## 4.1 Define Inside/Outside Interfaces

Router(config)# interface fa0/0

Router(config-if)# ip nat inside

Router(config)# interface fa0/1

Router(config-if)# ip nat outside

## 4.2 Static NAT

Router(config)# ip nat inside source static 192.168.1.10 203.0.113.10

## 4.3 Dynamic NAT

Router(config)# ip nat pool PUBLIC_POOL 203.0.113.20 203.0.113.25 netmask 255.255.255.248

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

Router(config)# ip nat inside source list 1 pool PUBLIC_POOL

## 4.4 PAT (NAT Overload)

Router(config)# ip nat inside source list 1 interface fa0/1 overload

# 5. Observations

## 5.1 NAT Translation Table

Pro  Inside global      Inside local       Outside global

tcp  203.0.113.1:1025  192.168.1.10:1025  8.8.8.8:80

tcp  203.0.113.1:1026  192.168.1.11:3201  8.8.8.8:80

tcp  203.0.113.1:1027  192.168.1.12:4502  8.8.8.8:80

# 6. Results and Analysis

| Feature | Static NAT | Dynamic NAT | PAT |
| --- | --- | --- | --- |
| Mapping | Permanent 1:1 | Dynamic 1:1 | Many:1 |
| Public IPs | 1 per host | Pool required | 1 total |
| Session Tracking | No | No | Yes (ports) |
| Best For | Servers | Mid-size corp | Home/ISP |

# 7. Conclusion

This lab successfully demonstrated all three forms of NAT. PAT (NAT Overload) emerged as the most efficient solution, enabling many private hosts to share a single public IP through port multiplexing. This is the fundamental technology that allows billions of devices behind home routers to access the internet using a single ISP-assigned address. NAT also provides a layer of security by hiding internal network structure.

# 8. References

- RFC 3022 — Traditional IP Network Address Translator

- Cisco NAT Configuration Guide

- Lammle, T. (2020). CCNA Certification Study Guide. Sybex.