COMPUTER NETWORKS LABORATORY

Lab Report

Lab 9 — Access Control Lists

Standard and Extended ACL Configuration

Lab File: lab_9.pkt

Course: Computer Networks

Date: March 2026

# 1. Objective

To configure and verify Standard and Extended Access Control Lists (ACLs) on Cisco routers, controlling network traffic and implementing basic security policies using Cisco Packet Tracer.

# 2. Background Theory

An ACL is an ordered set of permit/deny rules that a router uses to filter network traffic. ACLs are applied to router interfaces to control inbound or outbound traffic flow.

## 2.1 ACL Types

| Type | Number Range | Match Criteria |
| --- | --- | --- |
| Standard IPv4 | 1–99 | Source IP address only |
| Extended IPv4 | 100–199 | Source/Dest IP, Protocol, Port |
| Named Standard | N/A | Source IP (named format) |
| Named Extended | N/A | Full match criteria (named) |

## 2.2 ACL Processing Rules

- Rules are evaluated top-to-bottom — first match wins.

- An implicit deny all exists at the end of every ACL.

- ACLs must be applied to an interface and direction (in/out) to take effect.

## 2.3 Placement Guidelines

- Standard ACLs: Place close to the destination (matches source IP only).

- Extended ACLs: Place close to the source (matches detailed criteria).

# 3. Security Policy

- PC0 (Finance) must NOT access PC2 (Guest Network).

- PC1 (IT) can access all networks.

- HTTP traffic from PC2 is permitted; Telnet from PC2 is denied.

# 4. Configuration

## 4.1 Standard ACL — Deny PC0 from Guest Network

Router(config)# access-list 10 deny 192.168.1.0 0.0.0.255

Router(config)# access-list 10 permit any

Router(config)# interface g0/1

Router(config-if)# ip access-group 10 out

## 4.2 Extended ACL — Block Telnet, Allow HTTP from PC2

Router(config)# access-list 110 deny tcp 192.168.3.0 0.0.0.255 any eq 23

Router(config)# access-list 110 permit tcp 192.168.3.0 0.0.0.255 any eq 80

Router(config)# access-list 110 permit ip any any

Router(config)# interface g0/0

Router(config-if)# ip access-group 110 in

# 5. Observations & Results

## 5.1 Traffic Test Results

| Source -> Destination | Protocol | Result |
| --- | --- | --- |
| PC0 -> PC1 | ICMP | Permit |
| PC0 -> PC2 | ICMP | Deny (ACL 10) |
| PC2 -> Server (HTTP) | TCP 80 | Permit |
| PC2 -> Server (Telnet) | TCP 23 | Deny (ACL 110) |

## 5.2 ACL Match Counters

access-list 10

deny 192.168.1.0 0.0.0.255 (15 matches)

permit any (42 matches)

# 6. Conclusion

This lab demonstrated the configuration and verification of Standard and Extended ACLs. ACLs are a fundamental security tool in Cisco environments. Proper placement — close to source for extended, close to destination for standard — is critical for effective filtering. The implicit deny-all requires careful planning of permit statements to avoid blocking legitimate traffic.

# 7. References

- Cisco IOS Security Configuration Guide — ACLs

- RFC 3704 — Ingress Filtering for Multi-homed Networks

- Odom, W. (2020). CCNA 200-301 Official Cert Guide, Volume 2. Cisco Press.